Forster Thermfix Vario 45mm & 60mm (E30 + E60, EI30 + EI60 + EI90)

LSystem Manufacturer/Provider:

Forster	Rohr & Profiltechnik AG Steel Profile Systems, Arbon Switzerland.

Manufactured by:

Manufactured by Forster approved Fabricators, list of fabricators available from above contact.

Product reference:

Forster Thermfix Vario 45mm or 60mm is a thermally insulated, fully drained dry/pressure glazed

curtain walling system suitable for non fire, fire integrity only and fire integrity/insulation applications.

Forster Thermfix Vario has an elevation face width of both 45mm and 60mm, with various profile box

depths of between 50mm to 150mm depending on the size and application of the curtain wall to be

designed.

Glazing thickness ranges from between 6mm and 60mm, with the glass/panels held by glazing pads

and the pressure gaskets.

Thermfix Vario SV has been specifically designed for sloped glazing applications, and has a glass/infill

thickness of between 30mm to 37mm.

Thermfix Vario offers several differing types of external capping details, both in material choice and in

Visual profile.

General Use, Vertical (Non fire & up to EI90):

Thermfix can be used for all curtain walling requirements both internally for screens and atria and

externally for screens/staircases and building facades, and can be non fire rated or fire rated for

integrity and/or insulation up to EI90 dependant on relevant test data.

All fire test data complies with BS476 or BS-EN 1634.

Thermfix can incorporate non fire rated flush fitting windows, non fire and fire integrity &/or insulation

single doors, double doors or double acting doorsets depending on test data.

General Use, Sloped (Non fire):

For sloped roof applications Thermfix Vario SV is to be used as this is specially designed for this type

of application. This system allows the inclusion of manual and automatic opening roof vents, and can

be used in conjunction with normal Thermfix Vario vertical applications. Thermfix Vario SV is not for

use in fire rated applications.

Inclines for sloped curtain walling must be sufficient to allow easy drainage, and therefore Vario SV

can be used for inclines between 15 and 60 degrees from the horizontal position.

General Note:

For most applications it is required that structural load calculations are required to determine the

internal box profile section to be used. The size of this box will be determined by issues such as

spans, tie-back locations and wind load factors. Please consult with the fabricator, or, a representative

of Forster who can assist with these calculations.

Physical test data:

Thermfix Vario complies with prEN 13830 product standard, thereby fulfilling the requirements of CE

marking, and has been tested to achieve the following standards:

Water tightness EN 12154/ENV 13050, static RE1200, Dynamic 313Pa/938Pa

Air permeability EN 12152 class AE

Resistance to wind load EN 13116, Design load 1.25KN/m2, safety load 1.875Kn/M2

Impact resistance EN 14019, class E5 / I5

Fire tested and assessed to BS 476 / BS EN 1634

Thermal transmittance EN 12412-2, Uf = 1.5 – 2.4 W/(m2 k)

Acoustic insulation up to Rw,r = 45 dB

Ballistic testing to M1 –  M3 standard

Materials:

Steel

Profiles are supplied in standard black, Pre-Galvanized (GV-VC yellow chromated) finish or galvanized

finish.

Stainless Steel

Profiles are available in mill finish Stainless, to 304 standard. Profiles can be finish polished to

various requirements

Construction Method:

Thermfix Vario is a thermally insulated mullion/transome construction, fabricated by a welded and/or

slotted arrangement. This is determined by size, application, use, or specifier/fabricators

recommendation.

The system weatherproofing is fitted to the steel parts of the construction, with the glass/panel fitted

onto stainless steel retainers in pre-selected segments. The stainless pressure bar including system

weather proofing will be fitted on the outside by means of stainless steel locking screws. Locking

screws in conjunction with the spacers guarantee a controlled locking pressure for the system weather

proofing.

Pre finish

Internal/External applications

Thermfix Vario frame sections can either be etched primed, Two-Pack primed, Zinc rich primed or hot

dip-galvanized prior to powder coating depending on requirements.

Finish:

Standard finish to be Polyester powder coated to RAL/BS……. .

Glazing details:

Fire integrity & insulation

Single or double glazed units can be accommodated for fire integrity & insulation requirements.

Minimum glass thickness will generally be from approx 6mm and a maximum of 60mm for single and

double glazed units.

Examples of fire integrity/insulation glass used in Forster fire tests for EI30 are;

16mm Schott Pyranova, 15mm Pilkington Pyrostop, or 16mm Contraflam N2.

For EI60 examples are 21mm Glaverbel Pyrobel,  23mm Pilkington Pyrostop or 23mm Contraflam N2.

EI90 application examples are Paraflam 90 and Pyrostop 90.

Please note that maximum area sizes do apply and may differ between suppliers.

Double glazed units have been tested and are available for external uses or for when a level of sound

reduction is required.

Other combinations are also available incorporating low `E` glasses and tints. Only glasses that have

been fully tested for fire integrity & insulation may be used.

Internal/External glazing applications

Please note that for external fire integrity & insulated glazing it is generally thicker than for internal

applications. For example, for EI30 Schott Pyranova is 15mm thick for internal applications and 19mm

thick for external applications, whilst for EI60 Pilkington Pyrostop is 21mm thick for internal use and

27mm thick for external applications.

Non fire

Single or double glazed units can be accommodated with generally a minimum glass thickness from

approx 6mm to a maximum of 60mm for single and double glazed units. Various combinations of

double/triple glazed units can be accommodated for either sound reduction or light transmission

requirements.

Panel details:

Fire integrity/insualtion

Panels can be constructed to achieve integrity and insulation, these need to be as a minimum, each wall thickness of 2mm steel and with a rockwool infill ( eg. EI30 minimum 120kg/m3).

As a guide, the panels can be welded to the face of the profiles by slug welding or by crush folding top

and bottom to produce a profile flange, welding to the profile in the glazing areas.

Note that the panel size/thickness/filling may change depending on the fire requirement, please

consult with Forster who can assist with these details.

Non Fire

Various constructed panel variations can be incorporated with the Presto system,  and these can be

made from steel faced two panel trays, flat steel/plastic sheets or timber panels.

Doors & windows within Thermfix:

Various combinations of doors and opening vents can be incorporated into the Thermfix curtain walling system. Combinations that can be utilized are;

Non fire non thermally broken Presto doors and windows

Fire integrity Presto doors

Thermally broken non fire rated Therm Clima/Tur doors and windows, and

Fire integrity and insulated Fuego Light doors.

Please refer to the specification details pertaining to each of the required systems in regard to profiles, ironmongery and test data etc.

Fire

When specifying and using fire integrity & insulated doors, certain ironmongery must be used to

comply with the fire test data. Those items must always be Forster hinges, Forster locking

mechanisms and (unless it is a double acting door) a tested door closer, usually from the Dorma/Geze

ranges.  To ensure compliance please contact our Forster Technical Advisor or distributor.

Frame fixing to structure:-

Fixing of curtain walling can be achieved in several ways dependant upon building design and in

which manner the curtain wall is to be retained to the structure.

The framing can be retained to floor levels, structural steel work, block/concrete and within timber

constructions by utilizing individually designed welded or screwed on fixing/clamp brackets from the

profile box to the adjoining structure, or by spigot joint methods.

When fixing into concrete/stone/brick type material it is recommended that fixings should

be at least 50mm in from an edge to prevent breakout of the structure/fixing.

Services and Advice:-

Fabrication/Technical

For fabrication or technical advice, a full processing binder is available in hardback or pdf forms, these

detail processing guidelines, techniques, detail drawings and the full range of profiles and fittings

available from Forster.

Architectural/Specification

A full specification service is available from Forster to architects and specifiers requiring assistance

with NBS specification writing. Please contact the Forster Technical Advisor in the UK or our UK

Distribution company.