Forster Unico (Door & Window system)

L

System Manufacturer/Provider:

Forster Rohr & Profiltechnik AG Steel Profile Systems, Arbon Switzerland.

Manufactured by:

Manufactured by Forster approved Fabricators, list of fabricators available from above contact.

Product reference:

Forster Unico is a complete thermally broken 100% recyclable non fire rated profile system with fittings

and accessories for flush fitting doors & opening vents.

Forster Unico has a product depth of 70mm for doors& screens with openable vents at 88mm overall, available in both in steel and stainless steel.

General Use

Doors

Can be used for single & double leaf single acting doors, both inward or outward opening. Unico

can also allow combination units of doors & vents with the same profiles, and can be incorporated

within the Thermfix Vario curtain wall system.

Physical test data:

Unico complies with prEN 14351-1 product standard, thereby fulfilling the requirements of CE marking

and has been tested to achieve the following standards:

Thermal insulation EN 10077-2, Uf > 2.0 W/(m2 K)

Windows

Water tightness EN 12208, classes 9A

Air permeability EN 12207, class 4

Resistance against windloading EN 12210, classes C4/B4 & C5/B5 depending on configuration

Accoustic insulation up to Rw 48dB depending on configuration

Doors

Water tightness EN 12208, class 7A (1-leaf), class 7A (2-leaf)

Air permeability EN 12207, class 2 (1-leaf), class 3 (2-leaf)

Resistance against windloading EN 12210, class C3 (1-leaf), class C3 (2-leaf) depending on configuration.

Acoustic insulation up to Rw 45dB depending on configuration

Materials:

Steel

Profiles are supplied with galvanized finish (ZM, layer thickness 7 µm).

Stainless Steel

Profiles are available in mill finish Stainless to 304 grade. Profiles can be finish polished to

various requirements.

Construction method

Doors are to of fully welded construction, spigot joints can be used for multiple screens

or combinations frames.

Fabrication Tolerances:

Frames, doors and windows are to be manufactured within the following tolerances, these are to be

+/- 1.5mm in height and width, and +/- 4mm diagonally.

Pre finish

Internal/External applications

Doors, Frames & screens can be etched primed, Two-Pack primed or Zinc rich primed prior to powder

coating.

Finish:

Standard finish to be Polyester powder coated to RAL/BS……. .

Glazing details:

Double glazed units can be accommodated by Unico profiles. Minimum glass thickness of 6mm

is recommended and the maximum glass thickness is 46mm using various standard Forster clip-on

bead designs in steel, stainless steel and aluminium. Glasses can be of toughened or laminated in

nature or of specific requirements for burglar resistant or bullet resistance.

Panel details:

Various constructed panel variations can be incorporated with this system,  and these can be

made from steel faced two panel trays, flat steel/plastic sheets or timber panels.

Ironmongery:

Doors

Unico doors are designed to utilize a full range of Forster hinges, which are available depending on

choice or application. Hinges are either screw fixed to the door/frame or are welded to the door/frame.

A full range of locking devices are available depending on application required, and include a range of

latch/deadlocks with and without two/three point locking along with a complimentary range of lever handles. A range of panic latching devices as well as an emergency escape range is also available, along with a range of flush bolt locking mechanisms. The Forster emergency panic bar systems can also be fitted to Unico doorsets including anti-panic function.

Various overhead door closers can be fitted to this range of doors including automatic opening, smoke

detection and selector controlled devices, available from companies such as Dorma or Geze.

Windows/Vents

Unico windows/vents are designed to generally utilize a specially designed and tested range of fittings

for top/side/bottom hung & tilt/turn mechanisms that has been designed by Roto for this system.

Accessories:

Because Unico can be used in a multitude of applications, various types of accessories can be

used. For example, electric lockstrikes or magnetic locks can be easily fitted for

security applications or full door entry solutions with automatic detection and opening.

Fixing of ironmongery/accessories:

Fixing of ironmongery and accessories is to be by either special Forster connection plates for locks or

machine screws wherever possible, drilled and tapped to size as per manufacturers stipulation

wherever possible.

Self tapping screws or nutserts may be used for retaining certain ironmongery accessories that are

non critical situations, such as roses or escutcheons.

With pull handles utilize concealed back to back fixings, or if a handle is fitted to one side then ensure

it is secured by machine screws. All other fixings to be as recommended by the supplying

ironmonger or by the fabricator.

Frame fixing to structure:-

Fixing of frames can be achieved in several ways. The first is by having the fixing screws/bolts through

one of the frame box profiles and into the structure, with the fixing either being covered by a cap or by

the glazing detail.  The second alternative is the use of a special fixing plate can be used to allow the

fixing of the frame through the central lattice area, which is then covered by the weather gasket.

Finally, a common detail is by attaching (screwing or welding) a bracket or strap to the frame

and then fixing through the strap/bracket into the structure, which is then covered by plasterboard or

render etc. When fixing into concrete/stone/brick type material it is recommended that fixings should

be at least 50mm in from an edge to prevent breakout of the structure/fixing.

Typical fixings include woodscrews, plug & screw combinations, expanding anchors, machine

screws/bolts, and chemical fix.

Services and Advice:-

Fabrication/Technical

For fabrication or technical advice, a full processing binder is available in hardback or pdf forms, these

detail processing guidelines, techniques, detail drawings and the full range of profiles and fittings

available from Forster.

Architectural/Specification

A full specification service is available from Forster to architects and specifiers requiring assistance

with NBS specification writing. Please contact the Forster Technical Advisor in the UK or our UK

Distribution company.